Installation Guide for Remote Boot PROM

=============================================

Introduction

============

There are two kinds of Remote Boot PROM used with NetWare networks.

The two types are referred to as follows:

. IPX Boot PROM

. ODI Boot PROM

It is important that you should know the type of Remote Boot PROM you

are using. Some procedures for setting up remote booting on your computer

are specific to the type of Remote Boot PROM you are using. For more

information about the Remote Boot PROM you are using, please refer to

your dealler.

Installation Procedures

=======================

A. IPX Boot PROM installation procedures :

1. Make a bootable diskette, place that diskette into floppy driver A.

2. Copy the required files to that diskette, for example, "HIMEM.SYS",

"EMM386.EXE", "SETVER.EXE", "COMMAND.COM" ....

3. Copy "IPX.COM" and "NETX.COM" files to that diskette.

4. Create "AUTOEXEC.BAT" and "CONFIG.SYS" files.

5. Login NetWare as supervisor.

6. Execute following commands :

MAP G:=SYS:SYSTEM  <Enter>

MAP F:=SYS:LOGIN  <Enter>

7. Change to the F drive, then Execute following command :

G:DOSGEN <Enter>

8. If your DOS version is 5.0 or higher, execute following command :

RPLFIX NET$DOS.SYS <Enter>

9. Copy "AUTOEXEC.BAT" file from the boot diskette to the LOGIN directory.

10. Turn on the diskless computer.

--- Sample CONFIG.SYS file ---

:

DEVICE=SETVER.EXE

LASTDRIVE=E

:

--- Sample AUTOEXEC.BAT file ---

:

IPX

NETX

F:LOGIN

:

B. ODI Boot PROM installation procedures :

1. Make a bootable diskette, place that diskette into floppy driver A.

2. Copy the required files to that diskette, for example, "HIMEM.SYS",

"EMM386.EXE", "COMMAND.COM" ....

3. Copy "LSL.COM", "AN16.COM", "IPXODI.COM", "NETX.EXE" (or "VLM.EXE"

and all VLM files) to that diskette.

4. Create "AUTOEXEC.BAT", "CONFIG.SYS" and "NET.CFG" files.

5. Login NetWare as supervisor.

6. Execute following commands :

MAP G:=SYS:SYSTEM <Enter>

MAP F:=SYS:LOGIN <Enter>

7. Change to the F drive, then Execute following command :

G:DOSGEN <Enter>

8. If your DOS version is 5.0 or higher, execute following command :

RPLFIX NET$DOS.SYS <Enter>

9. Copy "AUTOEXEC.BAT" file from the boot diskette to the LOGIN directory.

10. Turn on the diskless computer.

--- Sample CONFIG.SYS file ---

:

DEVICE=SETVER.EXE

LASTDRIVE=Z      --------> only for VLM

:

--- Sample AUTOEXEC.BAT file ---

LSL

AN16

IPXODI

NETX             ---------> or VLM

F:LOGIN

--- Sample NET.CFG file ---

Link Support

Buffers 8 1514

LINK DRIVER AN16

FRAME ETHERNET_802.3

FRAME ETHERNET_802.2

NETWARE DOS REQUESTER

FIRST NETWORK DRIVE = F