NDIS Driver Installation Guide for IBM LAN Server

=======================================================

Introduction

============

This document describes the procedure to install NDIS driver on IBM

OS/2 LAN Server for NE2000 Compatible Ethernet Adapter.

These drivers conform to the Network Driver Interface Specification

(NDIS 2.01) defined by Microsoft/3Com.

Installation Procedures

=======================

A. Installing the driver for IBM LAN Server 2.0 or 3.0 :

Before you start with the installation  process,  make sure that

OS/2 LAN Server  is properly installed.  Similarly, your adapter

should also be properly installed in your server.

1. Copy AN16.OS2 and AN16.NIF files from your Driver diskette

(\LANSRVR) to C:\IBMCOM\MACS.

2. Click the OS/2 LAN Service icon.

3. Select "Installation  and  configuration".  The IBM logo should

appear onscreen.

4. Click <OK>.

5. Select <Advance> and press <Enter>.

6. Select "Install or Configuration this Workstation."

7. Select server type, adding a server or domain controller.  The

"install or remove" selections should appear.

8. Select "Configure a component."

10. Click the "Configure" button.

11. Select "Configure workstation."

12. Select "NE2000 Compatible Ethernet Adapter" when prompted to

identify your Network Adapter. Then click the <Add> button.

13. Select IBM NETBIOS from the list and click the <Add> button.

14. Select <OK> to confirm all selections.

15. Follow screen instructions to complete the process, selecting

<OK> to accept default selections.

16. Installation is complete now. Reset your PC by turning it off

and on.

B. Installing the driver for IBM OS/2 LAN Server 4.0 :

Before you start with the installation  process,  make sure that

OS/2 LAN Server  is properly installed.  Similarly, your adapter

should also be properly installed in your server.

1. In OS/2 window, Click the "MPTS" icon.

2. The screen will display the IBM Log, press "OK".

3. Select "Install" button.

4. Insert the driver diskette, specify the driver path A:\LANSRVR

and press <Enter>.

5. When complete above step to load AN16.OS2 into OS/2 system,

then configure your system.

6. The same step as step 3, select "Configure" button.

7. The screen will display "Configure" dialog box and select "LAN

adapters and protocols" then press "configure" button.

8. The screen will display  "LAPS Configuration"  dialog box,  move

highlight to "NE2000 Compatible Ethernet Adapter" item in "Network

Adapter" window, and select "NetBios" protocol. Press "OK".

9. Installation is complete now. Reset your PC by turning it off

and on.