Driver Installation Guide for MS LAN Manager v2.x (OS/2)

==============================================================

Introduction

============

This document describes the procedure to setup LAN Manager 2.x OS/2

Workstation/Server driver for NE2000 Compatible Ethernet Adapter.

These drivers conform to the Network Driver Interface Specification

(NDIS 2.01) defined by Microsoft/3Com.

Installation Procedures

=======================

A. Suppose you install MS LAN Manager on a OS2 workstation for the

first time.

1. Execute the "SETUP.EXE" program that is provided by LAN Manager.

Upon completion of retrieval of the last workstation disk's

contents, you will be asked if you want to import additional

network drivers.  Answer "YES".

2. Insert the driver diskette into the floppy drive.

3. Follow the instructions on the screen. "SETUP.EXE" program will

retrieve NDIS driver programs from the Driver disk and put them

on the harddisk and directory where LAN Manager resides.

4. Reboot your computer and follow NDIS binding procedure to use

the NDIS driver programs.

B. If you have installed LAN Manager in your harddisk.

1. Execute the "SETUP.EXE" program.

2. Insert the driver diskette into the floppy drive.

3. On the screen, select 'Configuration', 'Network Drivers',

'other Drivers' function and input correct diskette driver name.

4. Follow the instructions on the screen. "SETUP.EXE" program will

retrieve NDIS driver programs from the driver disk and put them

on the harddisk and directory where LAN Manager resides.

5. Reboot your computer and follow NDIS binding procedure to use

the NDIS driver program.

--- Sample PROTOCOL.INI file ---

[PROTOCOL MANAGER]

DRIVERNAME = PROTMAN$

[NETBEUI_XIF]

Drivername = netbeui$

SESSIONS = 40

NCBS = 85

BINDINGS = "AN16_NIF"

[AN16_NIF]

DRIVERNAME = AN16$