****************************************

*    Installation LAN Server Driver    *

****************************************

1. Insert the driver disk into drive A: or B:.

2. In the window titled "Command Prompt", click the OS/2 Full Screen

icon.

3. Jump to the IBMCOM directory and execute the LAPS command. The LAN

adapter protocol setup menu will appear. There are five items:

Install, Configure, Remove, Exit, and Help.

4. Click the "Install" item.

5. Assign drive A: or B: with path \LANSERVE to install the

driver onto the hard disk under the Install Aditional Network Drivers

window.

6. After copy is complete, the message "LAPS Successfully Installed

the Network Driver CNet CN935 PCI Bus Ethernet Adapter" appears.

Then, click OK and return to the main menu.

7. Next, choose the "Config" button to configure the environment of the driver.

8. Click on the "Continue" item in "Configure Workstation".

9. In the "Configure Workstation", the window consists of Network Adapter,

Protocol, and Current Configuration. Choose the "Cnet CN935 PCI Bus

Ethernet Adapter" item in Network Adapter along with the appropriate

protocol types.  Finally, the Current Configuration displays all of

user's requirements. Check that all is correct, then click the OK button.

10. Return to the main menu, choose exit, and reboot.