SETUP.EXE :

This program is provided for users to configure CN935 adapters. To run

this program, make sure the diskette is in the appropriate disk drive.

The SETUP.EXE program is in the SETUP subdirectory. Change the directory

to SETUP. At the DOS prompt type:

SETUP <ENTER>

-------------------------------------------------------------------

NOTE: Run this program BEFORE network drivers are loaded. Otherwise

it is likely this program will interfere with these drivers and the

computer will hang and require re-booting upon Exit.

-------------------------------------------------------------------

The following commands are available from the SETUP menu.

* Custom Setup: This option allows the user to set the card to Auto

sense, BNC, TP half-duplex or TP full-duplex.

* View Current Configration: The user can see the settings for up to

four installed cards.

* Run Diagnostics: This is a quick test of basic adapter functions.

Included are Controller Reset Test, EEPROM Test, IRQ Channel Test,

and Internal/External Loopback Tests.

* Exit: Exit the SETUP program.