****************************************

*    Installation for PC-NFS Driver    *

****************************************

1. Prepare the QuikNDIS diskette which contains the drivers for PC-NFS.

The adapter driver can not be installed without it. Insert the

diskette into drive A: or B:.

2. Insert the adapter driver disk into the other drive.

3. Run the QuikNDIS command.

4. After entering the "NDIS Driver" menu, type "0" to choose the third

party driver.

5. Key in the drive and path of "PCNFS" to indicate the diskette and

directory of the stored source driver, i.e. A:\PCNFS.

6. After disk duplication is complete, the "CNet CN935 PCI Bus Ethernet

Adapter" message appears.

7. Add the statement listed below in CONFIG.SYS. The program will update

PROTOCOL.INI files automatically.

DEVICE=C:\LANMAN\E935.DOS

8. Reboot the workstation.