**********************************************************

*     Installation for NetWare v4.x Server LAN Driver	 *

**********************************************************

The server driver file is stored in the \NW_4X subdirectory.

Installation Procedure:

1. Power on the file server and execute the Novell SERVER.EXE program.

2. There are two methods to load LAN driver E935.LAN.

>>Method 1:

At the server's prompt, load LAN driver E935.LAN and bind it.

This will automatically load MSM.NLM and ETHERTSM.NLM. The general

way of using these parameters is:

LOAD <driver name> <PARAMETER NAME>

Example:

LOAD A:\NW_4X\E935.LAN SLOT=16 FRAME=Ethernet_802.2 NAME=E935_802.2

BIND IPX TO E935_802.2 NET=2

LOAD A:\NW_4X\E935.LAN SLOT=16 FRAME=Ethernet_802.3 NAME=E935_802.3

BIND IPX TO E935_802.3 NET=3

NOTE: This assumes that this diskette is in drive A:.

Driver specific parameters list  (as for NetWare standard parameters,

please see the NetWare user's manual):

SLOT=xx	     On a PCI machine use slot number from 16,

for first ethernet device, and upwards.

TP 	     selects TP port

BNC	     selects BNC port

AUTODETECT      enables autodetection of media type when driver

is loading. Note that this ONLY! works at

initialization time, i.e. when the driver first

loads for a specific network card. (default)

TP_NO_LINK_TEST selects twisted pair connection, but disables

the LINK test for specific HUBs that do not

support LINK.

TP_FULL_DUPLEX  selects twisted pair connection to work in

full duplex mode.

BIOS16	     instructs the driver to access the PCI BIOS

using 16-bit real mode calls, instead of the

PCI32 mechanism.

This is not recommended, and should be used

ONLY! on motherboards that have a known problem

in the PCI BIOS implementation.

>>Method 2:

a. The installation files (i.e. E935.LAN & E935.LDI) are stored in the

\NW_4X subdirectory.

b. At the server's prompt type: LOAD INSTALL

c. When the install program comes up, choose the menu item.

d. Install the LAN driver E935.LAN according to the NetWare manual,

or use the on-line instructions.

NOTE: Please refer to the NetWare 4.x Installation Manual for details.

3. The server program will load and execute the E935.LAN driver module.

Please refer to the NetWare v4.x Installation Manual for more information

on making an AUTOEXEC.NCF batch file to load the E935.LAN module.