Personal NetWare 1.x Driver

***************************

(Drivers for Personal NetWare 1.x are stored in the \DOS_ODI subdirectory.)

INSTALLATION PROCEDURE

1)   Insert CNet's CN40BC PC Card Ethernet adapter into your

computer's Type II PC card slot.

2)   Connect the Media Coupler to the adapter's 15-pin connector.

3)   Connect the network cable to the RJ-45 or BNC connector on the

Media Coupler. (Refer to the User's Manual for exact instructions).

4)   Insert this Setup & Driver Diskette into the appropriate floppy

disk drive. (In the following examples, we will use drive "A:".)

5)   Make a directory on your hard disk (for example, C:\COPEN)

to hold the PC Card's setup and enabling utilities.

To make a directory, type:

MD C:\COPEN

To change to this directory, type:

CD C:\COPEN

6)   Copy the PowerNIC PC Card Ethernet adapter's enabling files

from the Setup & Driver Diskette to the "C:\COPEN" directory

of your hard disk.

To copy these files, type:

COPY A:\SETUP.EXE C:\COPEN

COPY A:\COPEN.EXE C:\COPEN

7)   Run the SETUP.EXE program and choose "Run Diagnostics"

to diagnose the CN40BC card.

If the Cable Loopback Test fails, please make sure that the Media

Coupler and network cable are connected.

8)   We strongly recommend that you select 'Y', when the SETUP

program requests "Do you want to access the network after the

PC reboots? (Y/N)". Selecting "Y" inserts the command line

"DEVICE=C:\COPEN\COPEN.EXE /IO=xxx /IRQ=yy /O"

into the C:\CONFIG.SYS file.

After this, each time when the computer boots the PowerNIC PC

Card Ethernet adapter will automatically be enabled.

(Remember, the adapter must be enabled before it can run

LAN drivers.)

9)  To install Personal NetWare, copy driver files to the Personal

NetWare directory.

At the DOS prompt, type the following commands:

COPY A:\DOS_ODI\E200P.INS C:\NWCLIENT

COPY A:\DOS_ODI\E200P.COM C:\NWCLIENT

10)  (a) Run Personal NetWare's SETUP.EXE to enter the main menu.

(b) Select the "Primary Network Interface Card" item, and press ENTER to

select "CNet CN40BC PC Card".

(c) Return to the main menu to select "Configure Primary Interface Card"

to configure the "INT", "PORT" and "Media Frame Type(s)".

Note: 1. The "INT" value MUST be the same as that of

"/IRQ=" option (yy) in the COPEN.EXE command line

from step 8).

2. The "PORT" value MUST be the same as that of

"/IO=" option (xxx) in the COPEN.EXE command line

from step 8).

(d) At the main menu, press "Save Changes and Exit".

11)  Execute STARTNET.BAT.