****************************************************

*                                                  *

*  CNet CN40BC NDIS 2 Device Driver Installation   *

*                                                  *

****************************************************

>> THIS FILE CONTAINS INSTRUCTIONS FOR BOTH DOS AND OS/2 ENVIRONMENTS. <<

>> THE OS/2 INSTRUCTIONS FOLLOW THE PROCEDURE FOR DOS.                 <<

**************************************************************************

Microsoft LAN Manager 2.1 Driver for DOS

========================================

INSTALLATION PROCEDURE

A. CN40BC PC Card Ethernet Adapter Enabler Program

1)   Insert CNet's CN40BC PowerNIC(tm) PC Card Ethernet adapter into your

computer's Type II PC Card slot.

2)   Connect the Media Coupler to the adapter's 15-pin connector.

3)   Connect the network cable to the RJ-45 or BNC connector on the

Media Coupler. (Refer to the User's Manual for exact instructions).

4)   Insert this Setup & Driver Diskette into the appropriate floppy

disk drive. (In the following examples, we will use "A:".)

5)   Make a directory on your hard disk (for example, C:\COPEN)

to hold the PC Card's setup and enabling utilities.

To make a directory, type:

MD C:\COPEN

To change to this directory, type:

CD C:\COPEN

6)   Copy the CN40BC PC Card Ethernet adapter's enabling files

from the "CNet Setup & Driver Diskette" to the "C:\COPEN"

directory of your hard disk.

To copy these files, type:

COPY A:\SETUP.EXE C:\COPEN

COPY A:\COPEN.EXE C:\COPEN

7)   Run the SETUP.EXE program and choose "Run Diagnostics"

to diagnose the CN40BC card.

If the Cable Loopback Test fails, please make sure that the Media

Coupler and network cable are connected.

8)   We strongly recommend that you select 'Y', when the SETUP

program requests "Do you want to access the network after the

PC reboots? (Y/N)". Selecting "Y" inserts the command line

"DEVICE=C:\COPEN\COPEN.EXE /IO=xxx /IRQ=yy /O"

into the C:\CONFIG.SYS file.

After this, each time when the computer boots, the CN40BC PC

Card Ethernet adapter will automatically be enabled.

(Remember, the adapter must be enabled before it can run LAN

drivers.)

B. LAN Manager 2.1 for DOS Installation

9)   Insert this Setup Diskette into the floppy disk drive.

10)  Select "Other Driver" when invoking C:\LANMAN.DOS\SETUP.EXE

to install the NDIS driver, and specify the drive in which

this diskette is inserted.

11)  Check the PROTOCOL.INI file to make sure that the INTERRUPT

and IOBASE agree with the settings on the LAN card.

Note that IOBASE MUST be the same as that of the "/IO=" option (xxx)

in the COPEN.EXE command. And the INTERRUPT value MUST be the same

as that of the "/IRQ=" option (yy).

12)  Make sure the sequence of command lines in the C:\CONFIG.SYS

file is similar to the example below:

:           :         :

[DEVICE=Socket Services command]

[DEVICE=Card Services command]

[DEVICE=resource manager]

:           :         :

DEVICE=C:\COPEN\COPEN.EXE /IO=xxx /IRQ=yy /O

:           :         :

DEVICE=C:\LANMAN.DOS\DRIVERS\Ethernet\E200P\E200P.DOS

:           :         :

***************************************************************************

LAN Manager for OS/2 Installation Guide

============================================

INSTALLATION PROCEDURE

A. CN40BC PC Card Ethernet Adapter Enabler Program

1)   Insert CNet's CN40BC PowerNIC(tm) PC Card Ethernet adapter into your

computer's Type II PC Card slot.

2)   Connect the Media Coupler to the adapter's 15-pin connector.

3)   Connect the network cable to the RJ-45 or BNC connector on the

Media Coupler. (Refer to the User's Manual for exact instructions).

4)   Insert this Setup & Driver Diskette into the appropriate floppy

disk drive. (In the following examples, we will use "A:".)

5)   Make a directory on your hard disk (for example, C:\COPEN)

to hold the PC Card's setup and enabling utilities.

To make a directory, type:

MD C:\COPEN

To change to this directory, type:

CD C:\COPEN

6)   Copy the CN40BC PC Card Ethernet adapter's enabling files from the

"CNet Setup & Driver Diskette" to the "C:\COPEN" directory of your

hard disk. To copy the file, type:

COPY A:\COPEN.OS2 C:\COPEN

B. LAN Manager for OS/2 Installation

7)   In the "Group Main" window, click the OS/2 "Full Screen" icon.

The full screen shell prompt will appear.

8)   Change the hard disk directory from the root to subdirectory \LANMAN.

9)   Key in the SETUP command (SETUP.EXE) in the LANMAN subdirectory.

10)  In the "Configuration" menu, choose "Network Drivers". The "Workstation

Configuration" window will appear.

11)  Click the "Add New Config" button.

12)  To select the driver for installation, click "Other Driver" in the

"Network Adapter Drivers" field.

13)  Assign the drive, A: or B:, without path in "Copy Driver or Protocol

Files" window. The "CNet CN40BC Ethernet PC Card" driver will be

copied to the hard disk. Then, click the "OK" button.

14)  In the "Network Protocols for CNet CN40BC Ethernet PC Card" window,

set the network protocol to be Netbeui (or other desired protocol,

such as TCP/IP) and click OK.

15)  Save the configuration and installation settings.

16)  Please make sure to add an extra statement in the CONFIG.SYS file

before the statement for the E200P.OS2 driver. The format of the

statement sequence should be similar to the example below:

:           :         :

DEVICE=C:\COPEN\COPEN.OS2 /IO=xxx /IRQ=yy /O

:           :         :

DEVICE=C:\LANMAN\DRIVERS\Ethernet\E200P\E200P.OS2

:           :         :

Note: "xxx" MUST be the same as the IOBASE value, and "yy" MUST be

the same as the INTERRUPT value in the [E200P_NIF]

section of the PROTOCOL.INI file of the \LANMAN subdirectory.

17)  Reboot.