Novell LAN WorkPlace for DOS Driver

***********************************

INSTALLATION PROCEDURE

1)   Insert CNet's CN40BC PC Card Ethernet adapter into your

computer's Type II PC card slot.

2)   Connect the Media Coupler to the adapter's 15-pin connector.

3)   Connect the network cable to the RJ-45 or BNC connector on the

Media Coupler. (Refer to the User's Manual for exact instructions).

4)   Insert this Setup & Driver Diskette into the appropriate floppy

disk drive. (In the following examples, we will use "A:".)

5)   Make a directory on your hard disk (for example, C:\COPEN)

to hold the PC Card's setup and enabling utilities.

To make a directory, type:

MD C:\COPEN

To change to this directory, type:

CD C:\COPEN

6)   Copy the PowerNIC PC Card Ethernet adapter's enabling files

from the Setup & Driver Diskette to the "C:\COPEN" directory

of your hard disk.

To copy these files, type:

COPY A:\SETUP.EXE C:\COPEN

COPY A:\COPEN.EXE C:\COPEN

7)   Run the SETUP.EXE program and choose "Run Diagnostics"

to diagnose the CN40BC card.

If the Cable Loopback Test fails, please make sure that the Media

Coupler and network cable are connected.

8)   We strongly recommend that you select 'Y', when the SETUP

program requests "Do you want to access the network after the

PC reboots? (Y/N)". Selecting "Y" inserts the command line

"DEVICE=C:\COPEN\COPEN.EXE /IO=xxx /IRQ=yy /O"

into the C:\CONFIG.SYS file.

After this, each time when the computer boots the PC

Card Ethernet adapter will automatically be enabled.

(Remember, the adapter must be enabled before it can run

LAN drivers.)

B. Install Novell LAN WorkPlace

9)   Use a text editor to modify C:LANWP.BAT as follows:

C:\yesno "Do you want to load the networking software? [y/n] "

if errorlevel 1 goto noload

PATH C:\LANWP4\BIN;%path%

C:\lsl.com

>> C:\ne2000.com  << ---  replace NE2000.COM with E200P.COM

C:\ipxodi.com

C:\netx.com

...

...

break on

:noload

10)  Next, use the text editor to modify C:NET.CFG like this:

>> Link Driver NE2000   << --- replace NE2000 with E200P

Protocol IPX 0 Ethernet_802.3

Frame Ethernet_802.3

Frame Ethernet_II

PORT    300  << --- This value MUST be the same as that

of "/IO=" option (xxx) in the

COPEN.EXE command line from step 8).

INT     5    << --- This value MUST be the same as that

...                 of "/IRQ=" option (yy) in the

...                 COPEN.EXE command line from step 8).

11)  At the DOS prompt, type the following command:

Copy A:\DOS_ODI\E200P.COM C:\

12)  Run LANWP.BAT