****************************************************

*                                                  *

*  CNet CN40BC NDIS 2 Device Driver Installation   *

*                                                  *

****************************************************

Banyan VINES 4.10 Driver

========================

INSTALLATION PROCEDURE

A. CN40BC PC Card Ethernet Adapter Enabler Program

1)   Insert CNet's CN40BC PowerNIC(tm) PC Card Ethernet adapter into your

computer's Type II PC Card slot.

2)   Connect the Media Coupler to the adapter's 15-pin connector.

3)   Connect the network cable to the RJ-45 or BNC connector on the

Media Coupler. (Refer to the User's Manual for exact instructions).

4)   Insert this Setup & Driver Diskette into the appropriate floppy

disk drive. (In the following examples, we will use "A:".)

5)   Make a directory on your hard disk (for example, C:\COPEN)

to hold the PC Card's setup and enabling utilities.

To make a directory, type:

MD C:\COPEN

To change to this directory, type:

CD C:\COPEN

6)   Copy the CN40BC PC Card Ethernet adapter's enabling files

from the "CNet Setup & Driver Diskette" to the "C:\COPEN"

directory of your hard disk.

To copy these files, type:

COPY A:\SETUP.EXE C:\COPEN

COPY A:\COPEN.EXE C:\COPEN

7)   Run the SETUP.EXE program and choose "Run Diagnostics"

to diagnose the CN40BC card.

If the Cable Loopback Test fails, please make sure that the Media

Coupler and network cable are connected.

8)   We strongly recommend that you select 'Y', when the SETUP

program requests "Do you want to access the network after the

PC reboots? (Y/N)". Selecting "Y" inserts the command line

"DEVICE=C:\COPEN\COPEN.EXE /IO=xxx /IRQ=yy /O"

into the C:\CONFIG.SYS file.

After this, each time when the computer boots, the CN40BC PC

Card Ethernet adapter will automatically be enabled.

(Remember, the adapter must be enabled before it can connect

to the network.)

B. NDIS Driver Installation

9)   Copy the E200P.DOS driver to the VINES directory that contains the file

BAN.EXE.

10)  Create a LANMAN subdirectory under the root, and copy the PROTOCOL.INI

file to this directory.

11)  Add the following three lines to your CONFIG.SYS file:

(The COPEN.EXE MUST be in front of E200P.DOS command.

If the COPEN.EXE command line is already in C:\CONFIG.SYS,

move that here.)

DEVICE=C:\COPEN\COPEN.EXE /IO=xxx /IRQ=yy /O

DEVICE=<VINES directory>\PROTMAN.DOS

DEVICE=<VINES directory>\E200P.DOS

12)  Check the PROTOCOL.INI file to ensure:

- the IOBASE value must be the same as that of the "/IO="

option (xxx) in the COPEN.EXE command, AND

- the INTERRUPT value MUST be the same as that of the "/IRQ="

option (yy) in the command.

13)  Run PCCONFIG and configure the NDIS Ethernet driver. Enter the

interrupt level and fill in the Section Name "E200P_NIF"

in the "BINDINGS =" field.

14)  Make sure the sequence of command lines in the C:\CONFIG.SYS

file is in the correct order, like the sample below:

:           :         :

[DEVICE=Socket Services command]

[DEVICE=Card Services command]

[DEVICE=resource manager]

:           :         :

DEVICE=C:\COPEN\COPEN.EXE /IO=xxx /IRQ=yy /O

:           :         :

DEVICE=<VINES directory>\E200P.DOS

:           :         :