****************************************************

*                                                  *

*  CNet CN40BC NDIS 2 Device Driver Installation   *

*                                                  *

****************************************************

Wollongong Driver

=================

INSTALLATION PROCEDURE

A. CN40BC PC Card Ethernet Adapter Enabler Program

1)   Insert CNet's CN40BC PowerNIC(tm) PC Card Ethernet adapter into your

computer's Type II PC Card slot.

2)   Connect the Media Coupler to the adapter's 15-pin connector.

3)   Connect the network cable to the RJ-45 or BNC connector on the

Media Coupler. (Refer to the User's Manual for exact instructions).

4)   Insert this Setup & Driver Diskette into the appropriate floppy

disk drive. (In the following examples, we will use "A:".)

5)   Make a directory on your hard disk (for example, C:\COPEN)

to hold the PC Card's setup and enabling utilities.

To make a directory, type:

MD C:\COPEN

To change to this directory, type:

CD C:\COPEN

6)   Copy the CN40BC PC Card Ethernet adapter's enabling files

from the "CNet Setup & Driver Diskette" to the "C:\COPEN"

directory of your hard disk.

To copy these files, type:

COPY A:\SETUP.EXE C:\COPEN

COPY A:\COPEN.EXE C:\COPEN

7)   Run the SETUP.EXE program and choose "Run Diagnostics"

to diagnose the CN40BC card.

If the Cable Loopback Test fails, please make sure that the Media

Coupler and network cable are connected.

8)   We strongly recommend that you select 'Y', when the SETUP

program requests "Do you want to access the network after the

PC reboots? (Y/N)". Selecting "Y" inserts the command line

"DEVICE=C:\COPEN\COPEN.EXE /IO=xxx /IRQ=yy /O"

into the C:\CONFIG.SYS file.

After this, each time when the computer boots, the CN40BC PC

Card Ethernet adapter will automatically be enabled.

(Remember, the adapter must be enabled before it can connect

to the network.)

B. NDIS Driver Installation

9)   Make a directory called NDIS on your hard disk.

10)  Copy E200P.DOS and PROTOCOL.INI from this diskette

to the C:\NDIS directory.

COPY A:\NDIS_20\E200P.DOS C:\NDIS

COPY A:\NDIS_20\PROTOCOL.INI C:\NDIS

11)  Insert the Pathway Setup Program diskette into the disk drive

and run the setup program by typing:

PWSETUP <enter>

12)  After the "Pathway Setup Program for DOS" window appears, choose

"Installation".

13)  In "Installation", pick "Pathway Run Time".  In "Pathway

Run Time", highlight "Quick Setup for NDIS".

14)  In the "Pathway Runtime Quick Setup for NDIS" window,

input the appropriate floppy disk drive letter as the source drive.

The destination should be "C:\PATHWAY".

Also, set the IP address and the number of subnet mask bits.

15)  In the "Driver NDIS and Kernel Configuation" window,

enter the path "C:\NDIS" to the device driver and the

PROTOCOL.INI file

16)  There is a screen editor for modifying the configuations with

respect to 10BASE, Hardware Interrupt, and Driver Name. To exit

the editor press the F4 key.

The I/O address MUST be the same as that of "/IO=" option (xxx)

in the COPEN.EXE command line from step 8) above.

The Interrupt line MUST be the same as that of "/IRQ="

option (yy) in the same command line.

17)  Next, use the select bar to change the software interrupt and

network frame type.

18)  The Pathway Setup program will copy all relevant files and modify

the CONFIG.SYS file as well as the AUTOEXEC.BAT file automatically.

Note: In the C:\CONFIG.SYS file, the COPEN.EXE command line

must precede the line E200P.DOS command. Below is a sample:

:           :         :

[DEVICE=Socket Services command]

[DEVICE=Card Services command]

[DEVICE=resource manager]

:           :         :

DEVICE=C:\COPEN\COPEN.EXE /IO=xxx /IRQ=yy /O

:           :         :

DEVICE=C:\NDIS\E200P.DOS

:           :         :

19)  Reboot.