****************************************************

*                                                  *

*  CNet CN40BC NDIS 2 Device Driver Installation   *

*                                                  *

****************************************************

Windows For Workgroups 3.1 Driver

=================================

INSTALLATION PROCEDURE

A. CN40BC PC Card Ethernet Adapter Enabler Program

1)   Insert CNet's CN40BC PowerNIC(tm) PC Card Ethernet adapter into your

computer's Type II PC Card slot.

2)   Connect the Media Coupler to the adapter's 15-pin connector.

3)   Connect the network cable to the RJ-45 or BNC connector on the

Media Coupler. (Refer to the User's Manual for exact instructions).

4)   Insert this Setup & Driver Diskette into the appropriate floppy

disk drive. (In the following examples, we will use "A:".)

5)   Make a directory on your hard disk (for example, C:\COPEN)

to hold the PC Card's setup and enabling utilities.

To make a directory, type:

MD C:\COPEN

To change to this directory, type:

CD C:\COPEN

6)   Copy the CN40BC PC Card Ethernet adapter's enabling files

from the "CNet Setup & Driver Diskette" to the "C:\COPEN"

directory of your hard disk.

To copy these files, type:

COPY A:\SETUP.EXE C:\COPEN

COPY A:\COPEN.EXE C:\COPEN

7)   Run the SETUP.EXE program and choose "Run Diagnostics"

to diagnose the CN40BC card.

If the Cable Loopback Test fails, please make sure that the Media

Coupler and network cable are connected.

8)   We strongly recommend that you select 'Y', when the SETUP

program requests "Do you want to access the network after the

PC reboots? (Y/N)". Selecting "Y" inserts the command line

"DEVICE=C:\COPEN\COPEN.EXE /IO=xxx /IRQ=yy /O"

into the C:\CONFIG.SYS file.

After this, each time when the computer boots, the CN40BC PC

Card Ethernet adapter will automatically be enabled.

(Remember, the adapter must be enabled before it can connect

to the network.)

B. Windows For Workgroups (WFW) Installation

9)   If there is no CN40BC NDIS driver installed on your hard

disk, select "Unlisted or Updated Network Adapter"

after clicking on the "Network" icon in the "Control

Panel" window. Specify "A:\NDIS_20" as the path and skip to

Step 11.

10)  If the CN40BC NDIS driver is already installed on the hard

disk, select "CNet CN40BC Ethernet PC Card" in the

network adapter list.

11)  Specify the correct configuration settings of the PC Card Ethernet

adapter, including IOBASE and INTERRUPT.

The IOBASE value MUST be the same as that of the "/IO=" option (xxx)

in the COPEN.EXE command line from Step 8) above.

The INTERRUPT value MUST be the same as that of the "/IRQ="

option (yy) in the same command line.

12)  Make sure the sequence of command lines in the C:\CONFIG.SYS

file is similar to the example below:

:           :         :

[DEVICE=Socket Services command]

[DEVICE=Card Services command]

[DEVICE=resource manager]

:           :         :

DEVICE=C:\COPEN\COPEN.EXE /IO=xxx /IRQ=yy /O

:           :         :

DEVICE=C:\Windows\E200P.DOS

:           :         :

13)  Exit and reboot the computer to put these changes into effect.

Note: If the driver was previously installed in WFW and you want to

re-install it from the diskette, the related OEM?.INF file in

the Windows\System directory must be deleted.