****************************************************

*                                                  *

*  CNet CN40BC NDIS 2 Device Driver Installation   *

*                                                  *

****************************************************

PC-NFS Driver for PCNFS version 4.0

===================================

INSTALLATION PROCEDURE

A. CN40BC PC Card Ethernet Adapter Enabler Program

1)   Insert CNet's CN40BC PowerNIC(tm) PC Card Ethernet adapter into your

computer's Type II PC Card slot.

2)   Connect the Media Coupler to the adapter's 15-pin connector.

3)   Connect the network cable to the RJ-45 or BNC connector on the

Media Coupler. (Refer to the User's Manual for exact instructions).

4)   Insert this Setup & Driver Diskette into the appropriate floppy

disk drive. (In the following examples, we will use "A:".)

5)   Make a directory on your hard disk (for example, C:\COPEN)

to hold the PC Card's setup and enabling utilities.

To make a directory, type:

MD C:\COPEN

To change to this directory, type:

CD C:\COPEN

6)   Copy the CN40BC PC Card Ethernet adapter's enabling files

from the "CNet Setup & Driver Diskette" to the "C:\COPEN"

directory of your hard disk.

To copy these files, type:

COPY A:\SETUP.EXE C:\COPEN

COPY A:\COPEN.EXE C:\COPEN

7)   Run the SETUP.EXE program and choose "Run Diagnostics"

to diagnose the CN40BC card.

If the Cable Loopback Test fails, please make sure that the Media

Coupler and network cable are connected.

8)   We strongly recommend that you select 'Y', when the SETUP

program requests "Do you want to access the network after the

PC reboots? (Y/N)". Selecting "Y" inserts the command line

"DEVICE=C:\COPEN\COPEN.EXE /IO=xxx /IRQ=yy /O"

into the C:\CONFIG.SYS file.

After this, each time when the computer boots, the CN40BC PC

Card Ethernet adapter will automatically be enabled.

(Remember, the adapter must be enabled before it can connect

to the network.)

B. NDIS Driver Installation

9)   Prepare the "QuickNDIS with drivers for PC-NFS" diskette and insert

it into the floppy diskette drive.

10)  Run the QUICKNDIS command.

11)  After the "NDIS Driver" menu appears, type "0" for the third

party driver.

12)  Input "A:\NDIS_20" when "Enter the device/path where the driver and

corresponding .NIF and PROTOCOL.INI files can be found" is displayed.

13)  Confirm the "Model=CNet CN40BC Ethernet PC Card.

Is this correct? (Y/N)" With "Y" <enter>.

14)  Make sure the IRQ, I/O address, and driver name are correct.

(Default values are 5, 0x300, and E200P$.)

'IOBASE=0x300' <enter> to accept

'Interrupt=5' <enter>

'DRIVERNAME = E200P$' <enter>

15)  Please make sure the COPEN.EXE command line precedes the

E200P.DOS command in the C:\CONFIG.SYS file. Follow the

sample below:

:         :        :

[DEVICE=Socket Services command]

[DEVICE=Card Services command]

[DEVICE=resource manager]

:         :        :

DEVICE=C:\COPEN\COPEN.EXE /IO=xxx /IRQ=yy /O

:         :        :

DEVICE=C:\LANMAN\E200P.DOS

:         :        :

where the xxx value MUST be the same as IOBASE value, and

yy MUST be the same as the Interrupt value in step 14).

16)  Insert a "NETBIND" command line if not present in the

C:\AUTOEXEC.BAT file. Example: C:\LANMAN\NETBIND.

17)  When done, reboot the workstation. The network card driver will be

loaded into memory.