****************************************************

*                                                  *

*  CNet CN40BC NDIS 2 Device Driver Installation   *

*                                                  *

****************************************************

LAN Server V3.0 Installation Guide

==================================

INSTALLATION PROCEDURE

A. CN40BC PC Card Ethernet Adapter Enabler Program

1)   Insert CNet's CN40BC PowerNIC(tm) PC Card Ethernet adapter into your

computer's Type II PC Card slot.

2)   Connect the Media Coupler to the adapter's 15-pin connector.

3)   Connect the network cable to the RJ-45 or BNC connector on the

Media Coupler. (Refer to the User's Manual for exact instructions).

4)   Insert this Setup & Driver Diskette into the appropriate floppy

disk drive. (In the following examples, we will use "A:".)

5)   Make a directory on your hard disk (for example, C:\COPEN)

to hold the PC Card's setup and enabling utilities.

To make a directory, type:

MD C:\COPEN

To change to this directory, type:

CD C:\COPEN

6)   Copy the CN40BC PC Card Ethernet adapter's enabling files from the

"CNet Setup & Driver Diskette" to the "C:\COPEN" directory of your

hard disk. To copy the file, type:

COPY A:\COPEN.OS2 C:\COPEN

B. LAN Server Installation

7)   Copy A:\NDIS_20\E200P.OS2 to C:\IBMCOM\MACS.

8)   Copy A:\NDIS_20\IBMLANSE\E200P.NIF to C:\IBMCOM\MACS\E200P.NIF.

9)   Invoke C:\IBMCOM\LAPS.EXE to install the network driver.

10)  There are several buttons including "Install", "Configure", "Remove",

"Exit" and "Help" in the "LAN Adapter and Protocol Support" window.

Choose the "Configure" button.

11)  In the "Configuration" subwindow, select the "Configure LAN Transports"

item and click on the "Continue" button.

12)  In the "Network Adapters" box of the "Configure Workstation" subwindow,

highlight the "CNet CN40BC Ethernet PC Card" and click on the "Add" button.

Now, the "Current Configuration" box shows the "CNet CN40BC Ethernet

PC Card...." message. The default PORT is 0x300 and IRQ is 5.

13)  Users can add the appropriate protocols in the "Protocols" box and the

chosen protocols will appear on the "Current Configuration" box as well.

14)  Add an extra statement in the CONFIG.SYS file before the statement

about the E200P.OS2 driver. The format of the statement sequence is

listed below. Note that the COPEN.OS2 command must precede the

E200P.OS2 command line.

:           :         :

DEVICE=C:\COPEN\COPEN.OS2 /IO=xxx /IRQ=yy /O

:           :         :

DEVICE=C:\IBMCOM\MACS\E200P.OS2

:           :         :

where xxx must match the PORT value and yy must be the same as

the IRQ value in step 12).

Note: Run SETUP.EXE in DOS if you want to know the card's status.