Files and Program operation description:

<1>. Introduction

DIAG9102 is a diagnostic program for DAVICOM DM9009/DM9102/DM9102A

based NIC. Now it supports the following NIC.

DM9009 NIC         : Support 10M/Full Half Duplex Mode

DM9102 NIC         : Support 10/100M Half/Full Duplex Mode

DM9102A NIC        : Support 10/100M Half/Full Duplex Mode

DM9102A+DM9801 NIC : Support 1M HomePNA and 10/100M Half/Full Duplex Mode

DM9102A+DM9802 NIC : Support 1M LongRun and 10/100M Half/Full Duplex Mode

HomeRun Only NIC   : Only Support 1M HomeRun

LongRun Only NIC   : Only Support 1M LongRun

In this diagnostic program, it has six items. Like as the following and

the detail description please refers <3>.

Adapter Information          Shows the configuration of NICs

Basic Diagnostics            Issues a basic diagnostic procedure

Advanced Diagnostics         Traffic test between two computers

Special Function             Do special function. It is only for DM9102A.

Config EEPROM                Modify setting in EEPROM

View the help file           Displays this file

Exit                         Exit this program

<2>. File Descriptions

DIAG9102.EXE         DM9102 diagnosis program

CUSTOMER.DAT         The customer's logo file

HELP.DOC             The help file

<3>. Diagnostic item description

*3-1 Adapter's Information: It shows the NIC's status and setting.

Adapter count      : How many NIC found on this system.

Adapter Type       : NIC type description

SROM Version       : SROM format version

EtherNet Address   : The NIC ethernet address

I/O Base Address   : System assigned I/O address

Mem Base Address   : System assigned memory address

INT Pin/Level      : System assigned interrupt vector

Vendor/Device ID   : Chip vendor and device ID

Sub System/Vendor  : Manufacturer ID

Adapter Handle     : An ID to access this NIC

Media Select       : User selects media mode. Only support by SROM

V4.01 or above

*3-2 Basic Diagnostics  : Performs a basic test, then displays a "Passed" or

"Failed" for every test item. If test pass, it just

show "PASS". If test failed, it will show the failed

item.

SROM ERR : It means SROM CRC check failed.

LOOKBACK : It means the internal lookback test failed.

NOTE: Please don't plug any cable when you do this test.

*3-3 Advanced Diagnostics: This mode lets users communicate between two

computers using a cross-over cable.

For different type NIC, it has different media modes.

DM9009 NIC        :

It has only one selections:

"10M Half-Duplex",

"10M Full-Duplex"

DM9102 NIC        :

DM9102A NIC       : It has five selections:

"10M Half-Duplex",

"10M Full-Duplex",

"100M Half-Duplex",

"100M Full-Duplex"

"Auto Detection"

DM9102A+DM9801 NIC:

DM9102A+DM9802 NIC: It has seven selections:

"10M Half-Duplex",

"10M Full-Duplex",

"100M Half-Duplex",

"100M Full-Duplex"

"Auto Detection",

"1Mbps Mode",

"1Mbps Mode EXT"

HomeRun only NIC  :

HomeRun only NIC  : It has two selections:

"1Mbps Mode",

"1Mbps Mode EXT"

First, users must connect two computers by a

cross-over cable. Then, users can execute the program

and select the media mode for each computer.

This mode supports four statistic counters.

According to results, users can know the NIC's

operating condition. We roughly divide those

counters into two (2) categories. One (1) is the

transmitted counter and transmitted error counter.

Another is the received counter and received error

counter.

*3-4 Special Function: This function is only for DM9102A, DM9102A/DM9801 and

DM9102A/DM9802 NIC. It did not support DM9009/DM9102

NIC. It has 4 function items to select. First is

reload the ethernet address from SROM. When user modify

the ethernet address in SROM, user can use this function

to reload data into MAC controller without power off

the system. Second, User can transmit a pause packet

with the maximum pause time(0ffffh) but it works on

100M full duplex and flow control enable. Third, it

is like as the second item but it send a pause packet

with the minimum time(0). Fourth, it sends a magic

packet.

User can set a ethernet address by press <F3> key.

It can wake the remote PC, if the remote PC supports

this function.

*3-5 Config EEPROM:    This function is only for DM9009, DM9102A, DM9102A/

DM9801 and DM9102A/DM9802 NIC. It did not support

DM9102 NIC. Now SROM format has two kinds. One is

V3.0 that is a compact definition. Another is V4.01

that is a complete definition for NIC.

NOTE: If you find something wrong, Please type "diag9102 -L" to generate

a NIC status file, name as "LOG.DAT". Then mail this file to

DAVICOM. It can help us to identify what problem happen. Thanks.