INTRODUCTION

This manual describes the installation information of E-LAN LAN

Adapter Packet Drivers for FTP's PC/TCP network softwares. At

first, let's have a brief look at the E-LAN LAN Adapters and

PC/TCP Softwares.

E-LAN Ethernet Adapter

E-LAN cards are Ethernet-based/IEEE 802.3 standard LAN interface

products which allows personal computers to communicate with each

other and share resources of laser printer, hard disks and common

data base, etc. The overall effect is a significant burst in your

office automation (OA) productivity. With each card, you can have

the advantages of a lot of features as below:

*Good quality and reliability.

*100% compliance with IEEE 802.3 and Ethernet CSMA/CD standard, 10

MegaBits per second data transfer rate.

*Supports Ethernet and Cheapernet (built-in transceiver drivers

more than 300 meters of Cheapernet cable).

*Hardware compatible with Novell's LAN cards.

*Supports PC-LAN, MS-NET, Novell's Advanced NetWare, FTP's PC/TCP

and APX LAN O.S.

*Remote boot ROM optional.

*Customer diagnostic program optional.

E-LAN-1000

*8-bit Ethernet LAN adapter

*IBM PC/XT/AT Compatible

*Compatible with NE-1000 and faster than it

E-LAN-2000

*16-bit Ethernet LAN adapter

*IBM AT/286/386 Compatible

*Compatible with NE-2000 and faster than it

FTP's PC/TCP

As you know, so far the TCP/IP is the best transport protocol for

worldwide host and internetworking data communication. It co-exits

with millions of mainframe, mini and microcomputer systems, as well

as with public telephone network, X.25 packet switching network,

or a variety of data networks.

PC/TCP is a collection of programs implementing the ARPA standard

TCP/IP protocol family for the IBM PC and compatibles. It supports

standard TCP/IP routing functions and subnetting, the use of IP

routers to connect multiple LANs together and the Internet domain

naming system. These programs let you connect your LAN to a TCP/IP

compatible host and run terminal sessions with TELNET, transfer

files with FTP, and use electronic mail systems with SMTP. You will

have convenient, reliable host-LAN connection.

*Standard ARPA TCP/IP protocol family: TELNET, FTP, TFTP, SMTP,

Finger, Whois, etc..

*Runs on IBM PC or compatibles under PC-DOS and MS-DOS.

*Supports standard subnetting, routing, domain hostname resolution.

*Supports Ethernet/Cheapernet connection with either E-LAN-1000 or

E-LAN-2000 card.

*E-LAN Ethernet interface driver for PC/TCP.

Note:

1. Besides PC/TCP, there are some related products available. All

of them can run with the E-LAN LAN Adapter's packet drivers.

These products are listed below:

PC/TCP options:

*InterDrive

*PC/TCP Plus

*PC/TCP Kernel

*PC/TCP Development Kit

InterDrive is FTP Software's client implementation of NFS. It runs

over PC/TCP V2.03 or the PC/TCP Kernel. It provides transparent,

fully functionally access to DOS applications and files stored on

NFS servers.

PC/TCP Development Kit provides the functionality of the

programming libraries and allows access to the higher-level

functions of FTP, TELNET and RLOGIN. Includes a Berkeley Sockets

emulation library. For use with Microsoft C 5.1.

PC/TCP Plus consists of PC/TCP with InterDrive for NFS accessing

and runs under DOS. PC/TCP Plus supports manu utilities for file

transfer, terminal emulation, mail, NFS<MI^>TM<D> file sharing

(through InterDrive), remote backup, printing and network testing,

as well as IP procedence, type-of-service, and basic & extended

security options. Besides, NetBIOS and Interrupt 14 terminal

emulation are options.

PC/TCP Kernel consists of PC/TCP without any applications. Included

are the underlying TCP, IP and UPD protocols. The PC/TCP Kernel can

be used to run custom or third party applications.

2. The present PC/TCP package that E-LAN LAN adapter support is

of Generic type with a type number of "210" (for PC/TCP, type# is

PC-210, for PC/TCP Plus it is I-210, while for PC/TCP Kernel is

K-210), and is to be run with E-LAN LAN adapter's packet

drivers. This Generic type can also be used to run with others LAN

adapters made by different vendors. However, it is necessary for

you to get their corresponding packet drivers for their LAN

adapters before you install it.

PREPERATION

Basically, to install the PC/TCP, you should follow the "PC/TCP

Generic Ethernet Driver Installation Guide" through the 6 major

steps. The contents described herein are part of the Device Driver

installation process only. However, there will be a slight

difference in Step 6 (Rebooting the PC) if the E-LAN LAN adapter

E-LAN-1000 or E-LAN-2000 is going to be used.

Before rebooting the PC, please copy the 'DRVCONF.COM',

'DRV2000.COM' and 'DRV1000.COM' files in the E-LAN adapter

packet driver diskettes to the same sub-directory which you have

created for "PC/TCP".

Referring to Item 1.7 in "PC/TCP Generic Ethernet Driver

Installation Guide", complete the following procedures before

running the PC/ping program or loading the kernel into memory.

H/W CONFIGURATION & SETTING

Check the jumper settings on your E-LAN LAN adapter to make sure

whether they are the same with the factory default settings (see

Table 1).

TABLE 1  FACTORY DEFAULT SETTINGS

1000		       2000

IRQ						3		       3

Base I/O Address				300H		       300H

Base Memory Address				CC00H		       C800H

If your jumper settings do not conform with Table 1 and there is

reason for you to change the default settings, adjust the driver's

I/O configuration table to match the hardware settings. In order

to do this, please run the 'DRVCONF.COM' installation program and

select the type of card to be used. The screen display for

selection of card is shown below:

PC/TCP Packet Driver Installation Program   Version X.XX

Please select one card (1. E-LAN1000  2. E-LAN2000):

Below are the screen displays for reconfiguring the E-LAN-1000 and

E-LAN-2000 respectively.

E-LAN-1000 PC/TCP Packet Driver Install Program

Current interrupt number: 6FH

Current IRQ=3,		I/O address=300H

I. Change Interrupt number? <y/n>y

Please input Interrupt number (60H-80H): 60H

II.Change IRQ number & I/O address? <y/n>y

1. IRQ=3,		I/O address = 300H

2. IRQ=2,		I/O address = 320H

3. IRQ=4,		I/O address = 340H

4. IRQ=5,		I/O address = 360H

5. IRQ=2,		I/O address = 300H

6. IRQ=3,		I/O address = 320H

7. IRQ=5,		I/O address = 340H

8. IRQ=4,		I/O address = 360H

9. IRQ=4,		I/O address = 300H

10.IRQ=5,		I/O address = 320H

11.IRQ=2,		I/O address = 340H

12.IRQ=3,		I/O address = 360H

please select one item (1-12): 1

Exit and save change data? <y/n>

E-LAN-2000 PC/TCP Packet Driver Install Program

Current interrupt number: 60H

Current IRQ=3,		I/O address=300H

I. Change Interrupt number? <y/n>y

please input Interrupt number (60H-80H): 60H

II.Change IRQ number & I/O address? <y/n>y

1. IRQ=3,	I/O address=300H	13. IRQ=3,	I/O address=300H

2. IRQ=2,	I/O address=320H	14. IRQ=2,	I/O address=320H

3. IRQ=4,	I/O address=340H	15. IRQ=4,	I/O address=340H

4. IRQ=5,	I/O address=360H	16. IRQ=5,	I/O address=360H

5. IRQ=2,	I/O address=300H	17. IRQ=2,	I/O address=300H

6. IRQ=3,	I/O address=320H	18. IRQ=3,	I/O address=320H

7. IRQ=5,	I/O address=340H	19. IRQ=5,	I/O address=340H

8. IRQ=4,	I/O address=360H	20. IRQ=4,	I/O address=360H

9. IRQ=4,	I/O address=300H	21. IRQ=4,	I/O address=300H

10.IRQ=5,	I/O address=320H	22. IRQ=5,	I/O address=320H

11.IRQ=2,	I/O address=340H	23. IRQ=2,	I/O address=340H

12.IRQ=3,	I/O address=360H	24. IRQ=3,	I/O address=360H

please select one item (1-24): 1

Exit install program? <y/n>

DRIVER LOADING & EXECUTION

Then run the 'DRV1000.COM'or 'DRV2000.COM' driver program. If there

is no error message shown, the driver is installed OK. Now, you may

proceed in testing the connection of your computer to the TCP/IP

network by running the "PC/ping" program. But before running the

other programs, you must first load  'ETHDRV', the kernel program,

into the system memory.

ERROR MESSAGE

Below are 4 kinds of "ERROR MESSAGES" with their respective Source

and Remedies:

1."CANNOT INITIALIZE ETHERNET CARD"

This error occurs when the hardware configuration and the driver

program have different settings. To correct this error, turn off

the computer and change the I/O Base and the IRQ of the hardware

to match with the driver's settings (refer to E-LAN-1000 and E-LAN-2000

manual); After the above operation, simply run the 'DRVCONF.COM'

to confirm whether the default settings of the Driver program is

correct or not.

2. "MEMORY TEST ON LAN CARD FAILS"

This error occurs when the LAN Card does not work normally. Recheck

the LAN Card (refer to the E-LAN LAN Adapter User's Manual).

3. "ETHERNET ADAPTER HARDWARE FAILURE"

This error occurs when the LAN Card does not work normally. Recheck

the LAN Card (refer to the E-LAN LAN Adapter User's Manual).

4. "FATAL ERROR. PLEASE CONTACT THE DISTRIBUTOR"

This error occurs when your LAN Adapter is not compatible with the

E-LAN LAN Adapter's Packet Driver programs.