���������������������������������������������������ͻ

�                                                   �

�         EN2K Ethernet Network Adapters            �

�  Copyright (C) 1994 By PDSL. All Rights Reserved  �

�            Release Date : Sep 20, 1994            �

�                      (940924)                     �

�                                                   �

���������������������������������������������������ͼ

INTRODUCTION

============

This is a reference for the EN2K User Diagnostic Version 1.00

(DIAGNOSE.EXE v1.00).

Important: This diagnostic program does not work if the EN2K card is

placed at the 8-bit slot.

There are three tests to verify the functionality of the network

adapter card. These tests are performed in sequence. However, if any one

test were to fail, the remaining tests will not be continued.

The On Screen Diagnostic window will show you the progress of

the three tests. The details are described as follow.

A) [Initialization]

In this test, all NIC registers are tested and verified.

B) [RAM Test]

This test perform Read-After-Write test on the memory module on the

network adapter card. It verifies the address, the data paths and

the memory modules separately.

C) [IRQ Test]

This test checks the configured IRQ# of the adapter card.

The test initiates the transmission of frames through the adapter

and then checks the interrupt line on the system.

Note that you should choose an interrupt line that DOES NOT

conflict with other adapters' settings, otherwise the system MAY HANG.

REMEMBER to terminate the adapter card.

======================== END =============================================