���������������������������������������������������ͻ

�                                                   �

�         EN2K Ethernet Network Adapters            �

�  Copyright (C) 1994 By PDSL. All Rights Reserved  �

�            Release Date : Aug 23, 1994            �

�                      (940823)                     �

�                                                   �

���������������������������������������������������ͼ

(1) INSTALLATION GUIDE

A) Installing the DOS ODI driver in a NetWare Lite v1.x environment

===================================================================

a) Follow the standard installation procedure detailed in

your NetWare Lite manual.

b) When prompted to select the "Network Interface Card

Name," select the option "*OTHER CARDS."

c) Insert the driver diskette, and press <ESCAPE>.

d) Select EN2K.COM from the menu.

e) Re-insert the NetWare Lite Install diskette and

complete the installation.

NOTE:

Should any problems be encountered, the EN2K.COM driver

can be manually added to the NetWare Lite startup procedure.

Simply select any other Interface card type (e.g. Novell

NE2000 ) when prompted during the NetWare Lite installation.

After this, a file called STARTNET.BAT is created by the

installation process in the C:\NWLITE sub-directory.

Complete the following steps:

a) Copy the EN2K.COM file from the driver diskette to

the C:\NWLITE sub-directory. Type

COPY A:\DOSODI\EN2K.COM C:\NWLITE

b) Edit the STARTNET.BAT file so that the second line

in the file states EN2K.COM and not the name of the

other Interface card type (e.g. NE2000.COM) that you

selected in the installation process.

c) Start NetWare Lite by typing STARTNET.BAT.

B) Installing the DOS ODI driver in a NetWare v3.1x environment

===============================================================

a) Make a directory in your hard disk, e.g. C:\NWCLIENT. Copy

the DOS ODI workstation files into this directory.

b) Copy the DOS ODI driver, EN2K.COM, and the NET.CFG from

the driver diskette into this directory.

c) To login to the server, run

LSL

EN2K

IPXODI

NETX

C) Installing the DOS ODI driver in NetWare v4.x environment

============================================================

Auto installation

-----------------

In a NetWare v4.0 environment, you may execute the NetWare

v4.0 Client INSTALL.EXE utility to install the DOS

Requester and generate the NET.CFG and CONFIG.SYS.

The INSTALL.EXE utility makes use of the INS file,

EN2K.INS, to prompt for configuration parameters that will

be copied into the NET.CFG.

Manual installation

-------------------

You may also manually execute the following steps:

a) Make a directory in your hard disk, e.g. C:\NWCLIENT. Copy

the DOS Requester files into this directory.

b) Copy the DOS ODI driver and the NET.CFG file from the

driver diskette into this directory.

c) Your NET.CFG file may look like this:

LINK DRIVER EN2K

NetWare DOS Requester

First Network Drive = F

d) In CONFIG.SYS, add the line "LASTDRIVE=Z"

e) Reboot the workstation.

f) To login to the network, run

LSL

EN2K

IPXODI

VLM

(2) STANDARD KEYWORDS SUPPORTED IN NET.CFG FILE

Two main keywords supported in the NET.CFG file are:

1. PORT

Valid PORT addresses are:

240h, 280h, 2C0h, 300h, 320h, 340h and 360h.

In most cases, it is UNNECESSARY to specify the PORT

address. The driver automatically reads the I/O base

address from the network adapter if this keyword is not

specified.

However, by specifying this keyword in the NET.CFG, you can

CHANGE the PORT address of your network adapter regardless

of the original setting. For example to change the I/O

base address to 2C0h, simply write:

LINK DRIVER EN2K

PORT 2C0

2. INT

Valid INT levels are:

2, 3, 4, 5, 10, 11, 12 and 15.

In most cases, it is UNNECESSARY to specify the INT level.

The driver automatically reads the hardware interrupt

number from the network adapter if this keyword is not

specified.

However, by specifying this keyword in the NET.CFG, you can

CHANGE the interrupt level of your network adapter

regardless of the original setting. For example to change

the hardware interrupt to 15, write:

LINK DRIVER EN2000

INT 15

3. CABLE

Valid cable types are : (1)   BNC (Thin Ethernet)

(2)   TPI (Twisted-pair)

This keyword is used to change the cable type of your

adapter. If the cable type is not specified, the preset cable

type on your adapter is effective.

For example, you may write in the NET.CFG file:

LINK DRIVER EN2K

CABLE TPI

(3)  MULTIPLE CARDS

If there are multiple EN2K boards installed in your

system, specify the port address so that the driver knows

which cards to initialize. Otherwise, the driver will

initialize the first EN2K board it locates.