���������������������������������������������������ͻ

�                                                   �

�         EN2K Ethernet Network Adapters            �

�  Copyright (C) 1994 By PDSL. All Rights Reserved  �

�            Release Date : Aug 23, 1994            �

�                      (940823)                     �

�                                                   �

���������������������������������������������������ͼ

Packet Driver Installation For EN2K Ethernet Adapter

----------------------------------------------------

The driver EN2KPD.COM can be used for the following EN2K Ethernet

adapters:

EN2K-TP

EN2K-CX

EN2K-COMBO

EN2KPD syntax:

EN2KPD -i <Packet_int_no> -b <io_addr> -q <int_level>  -u

-i	  Set software interrupt number

-b        Set I/O Base address

-q        Set hardware IRQ

-u 	  Unload driver

Example:

The following command line loads the EN2K Packet Driver at

software interrupt 60h, I/O address 300h, and hardware interrupt 3.

EN2KPD -i 0x60 -b 0x300 -q 3

If you just type:

EN2KPD

the packet driver program EN2KPD will search for the  default I/O address

and the default hardware interrupt.

After the Packet Driver is loaded, you can run the PC/TCP kernel

ETHDRV.EXE. The PC is now ready for PC/TCP communications.

To run PC/TCP applications, You need to load PC/TCP Kernel

("ETHDRV.EXE").

a. Refer to PC/TCP manual, Command Reference section,

Chapter 2.3.

b. At this point, you are ready to run PC/TCP applications

(e.g., 'ftp', 'telnet').

For example you may type

FTP 192.72.24.202

where 192.72.24.202 is a host address.

For information regarding the "FTP's PC/TCP Network

Software for DOS" packages, please contact:

FTP Software, Inc.

26 Princess Street

Wakefield, MA  01880

(617) 246-0900