2/1/1996

SCO UNIX SureCom EP325 MDI Driver

==================================================

The following files will be used in this installation:

Files          Contents

-----------    -----------------------------------------

Driver.o       EP325 MDI Driver Program

setup          Transfer dos format to unix format setup program

Master         Used by netconfig

System         Used by netconfig

Node           Used by netconfig

Space.c        Used by netconfig

space.h        Used by netconfig

lkcfg          Used by netconfig

AOF/r3e        Used by netconfig

1.  Installation Diskette in DOS format.

Put EP325 SCO UNIX Driver diskette to floppy A: and use doscp

command to copy EP325 SCO UNIX PCI Driver into UNIX's directories.

# cd /

# doscp A:setup  /setup

# chmod +x /setup

# ./setup

2.     Use netconfig utility

Avaiable options:

# netconfig

3.  To add adapters, select "Add New Lan Adapter" from "Hardware".

4.  Add Protocol to select SCO TCP/IP.

5.  Enter the internet Address of this interface    n.n.n.n

6.  Enter the netmask for this interface            (default 255.255.255.0)

7.  Enter the boardcast address for this interface  default: n.n.n.n

8.  Do you want to relink to kernel now ?           y

9.  Do you want this kernel to boot by default (y/n)    y

10. Do you want the kernel environment rebuilt (y/n)    y

11. Reboot unix

# shutdown -y -g0