******************************************************************************

*             Genius LAN GE2000III Ethernet Card -- SETUP Program            *

******************************************************************************

A. OVERVIEW

===========

The SETUP.EXE Program is used to configure and verify the correct

configuration and operation of an Genius LAN GE2000III series LAN

adapter before installing the network software.

B. BEFORE STARTING THE PROGRAM

==============================

1. Be sure you know the configuration of any adapters.

2. After turning off the PC's power, insert the adapter into the PC.

** Ethernet adapters must be on a properly terminated cable

(use a BNC " T " connector with two 50 Ohm terminators or

an AUI loopback connector).

3. Boot the PC to a DOS prompt.

** Memory resident (TSR) programs and device drivers should

not be loaded while the Diagnostic Program is active, so

you may need to temporarily rename your CONFIG.SYS and

AUTOEXEC.BAT files.  The Diagnostic Program is always run

without loading any network software.

C. STARTING THE PROGRAM

=======================

The Genius LAN GE2000III Ethernet card support Jumper,Jumperless and

Plug&Play modes. In Plug&Play mode, the card's configuration is

determined automatically by the Plug&Play system PC. You can also

use the SETUP program to change the card's configuration.

1. Insert the GE2000III driver diskette and change to the SETUP

subdirectory. In the DOS prompt, type GE3SETUP to run SETUP program,

like as below:

A:>GE3SETUP

2. Read the opening screen and press Enter.

3. Adapter configuration:

View Setup   - The setup program will auto detect and show the card's

configurations, such as I/O port, IRQ,... etc.

Manual Setup - Select Manual Setup to setup hardware configurations

that you want, such as I/O, IRQ, card mode, operating

mode, Full duplex, BootROM usage.

Save Configuration - Save the hardware configuration that be changed.

***** Warning *******

After you changed the hardware configuration, you must reset your

PC. The adapter will load new configurations that you changed from

EEPROM.

4. Adapter Diagnostic

Hardware Configuration - To view the hardware configurations.

Ethernet Card Test     - To test the adapters' function is correct.

Transmit/Receive Test  - To verify the adapter transeveiver is ok.

Cable Test             - To test cable is no fault.

D. SUPPORT

==========

If you have any additional questions regarding your Diagnostic

Program, contact your dealer or distributor technical support

department.

======================================================================