�����������������������������������������������������������������Ŀ

�        TCP/IP Packet Driver Installation for HE200 Adapter      �

�������������������������������������������������������������������

The TCP/IP packet driver must be installed prior to use the adapter

under the TCP/IP environment.  Since this driver takes only a few

thousand bytes, its installation is best done directly within your

AUTOEXEC.BAT.  The Highlight HE200 adapter card supports the way of

configuring the hardware parameters  by setting jumpers to match

with driver by directly assignment from DOS command prompt.

This is where things get a little technical. Because of the nature

of the TCP/IP standard,  all parameters passed  to the driver must

be specified in  C  [language]  style representation.   Using this

notation, e.g. the number "11" (in decimal) would be equivalent to

"0x0B" in hexadedimal (hex) or "013" in octal notation. Any number

results returned by the packet driver will be in the same notation.

Simply running  a packet driver  without listing any  accompanying

parameters will return the error messages.

The command line usage for the TCP/IP packet driver is:

[drive]> HE200PKT [-n] [-d] [-w] <packet_int_no> <int_level> <io_addr>

[-n] : The  "-n"  option  is used to convert IPX packets between

the  Ethernet II type 8137  encapsulation  used by  BYU's

PDSHELL IPX interface code and the 802.3-style encapsula-

tion normally used on Ethernet by NetWare servers, shells

and boot ROMs.  It will also  convert incoming type  8137

packets to type 8138 so that NetWare doesn't get confused.

[-d] : The "-d" option is used to delay the initialization of the

Ethernet card until the packet driver is used for the first

time.  See PROMBOOT.NOT for how to use -d and -n in a boot

PROM environment.

[-w] : The  "-w"  switch is used for Windows.  Install the packet

driver  BEFORE running MS-Windows.   This switch  does not

prevent Windows from swapping your network application out

of memory, it simply detects  when that has happened,  and

drops the packets on the floor.

<packet_int_no> :

Before installing the packet driver,  you have to choose a

software interrupt number in range  between 0x60 and 0x80.

Some of these interrupts  are used for other purposes,  so

your first choice  may not work.  See the  INTERRUPT USAGE

section of Ralf Brown's interrupt list extract (at the end

of this document ) between 0x60 and 0x80.

<int_level> :

Indicates the hardware interrupt being used by the network

card. The range will depend on the hardware designed, refer

to the hardware user's guide for details.

<io_addr> :

Indicates the base I/O address  for the network card.  The

value range will depend on the hardware designed, refer to

the hardware user's guide for details.

Following is a sample command line using the packet driver

through the command line options:

A> HE200PKT  0x66  3  0x300  ( for HE200 adapter )

INTERRUPT USAGE

������������������������������������������������������

This interrupt usage list for the range 0x60 through 0x80 is excerpted

from Ralf Brown's interrupt list.

��������������������������������������������������������������������Ŀ

� 60 -- -- reserved for user interrupt				�

� 60 -- -- FTP Driver - PC/TCP Packet Driver Specification		�

� 60 01 FF FTP Driver - DRIVER INFO					�

� 60 02 -- FTP Driver - ACCESS TYPE					�

� 60 03 -- FTP Driver - RELEASE TYPE 				�

� 60 04 -- FTP Driver - SEND PACKET					�

� 60 05 -- FTP Driver - TERMINATE DRIVER FOR HANDLE			�

� 60 06 -- FTP Driver - GET ADDRESS					�

� 60 07 -- FTP Driver - RESET INTERFACE				�

� 60 11 -- 10-NET - LOCK AND WAIT					�

� 60 12 -- 10-NET - LOCK						�

� 60 13 -- 10-NET - UNLOCK						�

� 60 20 -- FTP Driver - SET RECEIVE MODE				�

� 60 21 -- FTP Driver - GET RECEIVE MODE				�

� 60 24 -- FTP Driver - GET STATISTICS				�

� 61 -- -- reserved for user interrupt				�

� 62 -- -- reserved for user interrupt				�

� 63 -- -- reserved for user interrupt				�

� 64 -- -- reserved for user interrupt				�

� 65 -- -- reserved for user interrupt				�

� 66 -- -- reserved for user interrupt				�

� 67 -- -- LIM EMS							�

�	      ...							�

� 67 DE 00 Virtual Control Program Interface - INSTALLATION CHECK	�

�	      ...							�

� 68 01 -- APPC/PC							�

�	      ...							�

� 69 -- -- unused							�

� 6A -- -- unused							�

� 6B -- -- unused							�

� 6C -- -- system resume vector (CONVERTIBLE)			�

� 6C -- -- DOS 3.2 Realtime Clock update				�

� 6D -- -- VGA - internal						�

� 6E -- -- unused							�

� 6F -- -- Novell NetWare - PCOX API (3270 PC terminal interface)	�

� 6F 00 -- 10-NET - LOGIN						�

�	      ...							�

� 70 -- -- IRQ8 - AT/XT286/PS50+ - REAL-TIME CLOCK			�

� 71 -- -- IRQ9 - AT/XT286/PS50+ - LAN ADAPTER 1			�

� 72 -- -- IRQ10 - AT/XT286/PS50+ - RESERVED 			�

� 73 -- -- IRQ11 - AT/XT286/PS50+ - RESERVED 			�

� 74 -- -- IRQ12 - PS50+ - MOUSE INTERRUPT				�

� 75 -- -- IRQ13 - AT/XT286/PS50+ - 80287 ERROR			�

� 76 -- -- IRQ14 - AT/XT286/PS50+ - FIXED DISK			�

� 77 -- -- IRQ15 - AT/XT286/PS50+ - RESERVED 			�

� 78 -- -- not used							�

� 79 -- -- not used							�

� 7A -- -- Novell NetWare - LOW-LEVEL API				�

�									�

� 7A -- -- AutoCAD Device Interface					�

� 7B -- -- not used							�

� 7C -- -- not used							�

� 7D -- -- not used							�

� 7E -- -- not used							�

� 7F -- -- HDILOAD.EXE - 8514/A VIDEO CONTROLLER INTERFACE		�

� 7F -- -- HLLAPI (High-Level Language API)				�

� 80 -- -- reserved for BASIC					�

����������������������������������������������������������������������

=================== FILE TCPIP\INSTALL.DOC -- END OF FILE =================