This section provides two installation procedures: the first is for

NetWare 3.11. The procedure for 3.12 follows.

***********************************************

*  CNet PCI-Bus CN930 Driver Installation     *

*  > NetWare 3.11 ODI Server		      *

***********************************************

The NetWare 3.11 ODI server driver file is stored in the \NW_31X

subdirectory of the CN930 diskette.

1. Power on the file server, and execute Novell's SERVER.EXE program.

2. Load LSLENH.NLM first (this also automatically loads PATCHMAN.NLM).

At the server's prompt, type:

LOAD A:\NW_31X\LSLENH.NLM

3. Load LAN driver E930.LAN and bind it. This will automatically load

MSM31X.NLM and ETHERTSM.NLM. The general way of using these parameters

is:

LOAD <driver name> <PARAMETER NAME>

Example:

LOAD A:\NW_31X\E930.LAN FRAME=Ethernet_802.2 NAME=E930_802.2

BIND IPX TO E930_802.2 NET=2

LOAD A:\NW_31X\E930.LAN FRAME=Ethernet_802.3 NAME=E930_802.3

BIND IPX TO E930_802.3 NET=3

NOTE: This assumes that this diskette is in drive "A:".

4. The server program will load and execute the E930.LAN driver module.

Please refer to the NetWare 386 V3.11 Installation Manual for more

information to make an auto-execution batch file to load the E930.LAN

module.

NOTE:

1. Installing Multiple LAN Adapters:

The keyword "EtherID" is the ethernet address of a adapter. This

keyword is provided for multiple LAN adapters in a single server

by the driver E930.LAN.  So, add EtherID in LOAD commands. For

example:

LOAD A:\NW_31X\E930 EtherID=0080AD123456 FRAME=Ethernet_802.2 NAME=LAN_A

BIND IPX TO LAN_A NET=11

LOAD A:\NW_31X\E930 EtherID=0080AD123457 FRAME=Ethernet_802.3 NAME=LAN_B

BIND IPX TO LAN_B NET=22

2.  If you use the MONITOR.NLM shipped with NetWare 3.11, your server could

abend when viewing LAN statistics in MONITOR.NLM with E930.LAN driver.

We have provided the newer Novell MONITOR.NLM program in the driver

diskette to correct this problem.

***********************************************

*  CNet PCI-Bus CN930 Driver Installation     *

*  > NetWare 3.12 ODI Server		      *

***********************************************

The NetWare 3.12 ODI server driver file is stored in the \NW_31X

subdirectory of the CN930 diskette.

1. Power on the file server, and execute Novell's SERVER.EXE program.

2. Load the LAN driver E930.LAN and bind it (this also automatically loads

MSM31X.NLM and ETHERTSM.NLM). The general way of using these parameters

is:

LOAD <driver name> <PARAMETER NAME>

Example:

LOAD A:\NW_31X\E930.LAN FRAME=Ethernet_802.2 NAME=E930_802.2

BIND IPX TO E930_802.2 NET=2

LOAD A:\NW_31X\E930.LAN FRAME=Ethernet_802.3 NAME=E930_802.3

BIND IPX TO E930_802.3 NET=3

NOTE: This assumes that this diskette is in drive A.

3. The server program will load and execute the E930.LAN driver module.

Please refer to the NetWare 3.12 Installation Manual for more

information on how to make an auto-execution batch file to load the

E930.LAN module.

NOTE: Installing Multiple LAN Adapters:

The keyword "EtherID" is the ethernet address of a adapter. This

keyword is provided for multiple LAN adapters in a single server

by the driver E930.LAN.  So, add EtherID in LOAD commands. For

example:

LOAD A:\NW_31X\E930 EtherID=0080AD123456 FRAME=Ethernet_802.2 NAME=LAN_A

BIND IPX TO LAN_A NET=11

LOAD A:\NW_31X\E930 EtherID=0080AD123457 FRAME=Ethernet_802.3 NAME=LAN_B

BIND IPX TO LAN_B NET=22