***********************************************

*  CNet PCI-Bus CN930 Driver Installation     *

*  > Packet Driver			      *

***********************************************

The packet driver for PC/TCP is located in the \PCTCP subdirectory

of the CN930 diskette.

1. Run the E930PD.COM program by entering the following

at the DOS command line --

A:>\PCTCP\E930PD [options] <Packet_INT#>

(NOTE: This assumes the driver diskette is in drive A.)

EXAMPLE:

E930PD  0x60

--> Packet_INT# = 0x60

The only required parameter is the software interrupt. A packet

driver needs to have a software interrupt assigned to it for other

programs to access it.  The recommended interrupt 0x7e.  If you need

any of the options listed below, place them before the software

interrupt on the command line.

-i - Force driver to report itself as 802.3 instead of Ethernet II.

-d - Delayed initialization.  Used for diskless booting

-n - NetWare conversion.	Converts 802.3 packets into 8137 packets

-w - Windows hack, obsoleted by winpkt

-p - Promiscuous mode disable

-h - Help

-u - Uninstall

2. Execute the ETHDRV.EXE device program, which is included with the

FTP PC/TCP software package.

The Ethernet adapter should now work with FTP PC/TCP.