***********************************************

*  CNet PCI-Bus CN930 Driver Installation     *

*  > NetWare 4.x ODI Server		      *

***********************************************

The NetWare 4.x ODI server driver file is stored in the \NW_4

subdirectory of the CN930 diskette.

1. Power on the file server, and execute Novell's SERVER.EXE program.

2. Load the LAN driver E930.LAN. (Two different methods are shown here.)

METHOD 1 --

At the server's prompt, load the driver E930.LAN and bind it

(this also automatically loads MSM.NLM and ETHERTSM.NLM). The general

way of using these parameters is:

LOAD <driver name> <PARAMETER NAME>

Example:

LOAD A:\NW_4\E930.LAN FRAME=Ethernet_802.2 NAME=E930_802.2

BIND IPX TO E930_802.2 NET=2

LOAD A:\NW_4\E930.LAN FRAME=Ethernet_802.3 NAME=E930_802.3

BIND IPX TO E930_802.3 NET=3

NOTE: This assumes that this diskette is in drive A.

METHOD 2 --

a) At the server's prompt type: LOAD INSTALL

b) When the install menu appears, choose the menu item.

c) Install the LAN driver E930.LAN according to the NetWare manual,

or use the on-line instructions.

3. The server program will load and execute the E930.LAN driver module.

Please refer to the NetWare 4.x Installation Manual for more information

on making an auto-execution batch file to load the E930.LAN module.

NOTE: Installing Multiple LAN Adapters:

The keyword "EtherID" is the ethernet address of a adapter. This

keyword is provided for multiple LAN adapters in a single server

by the driver E930.LAN.  So, add EtherID in LOAD commands. For

example:

LOAD A:\NW_4\E930 EtherID=0080AD123456 FRAME=Ethernet_802.2 NAME=LAN_A

BIND IPX TO LAN_A NET=11

LOAD A:\NW_4\E930 EtherID=0080AD123457 FRAME=Ethernet_802.3 NAME=LAN_B

BIND IPX TO LAN_B NET=22