***********************************************

*  CNet PCI-Bus CN930 Driver Installation     *

*  > Windows for Workgroups 3.1               *

***********************************************

The DOS NDIS driver for Windows for Workgroups is located in the

\NDIS_20 subdirectory of the CN930 diskette.

1. From the Control Panel, click on the "Network" icon.

2. Click on the "Adapters" and then click on the "Add..." button.

3. If there is no CN930 NDIS driver installed on the hard disk, select

"Unlisted or Updated Network Adapter". Specify "A:\NDIS_20" as the path

(assuming this diskette is in drive A). Skip to Step 4.

4. Otherwise, select "CNet CN930 Series PCI Ethernet Adapters" in

the network adapter list.

5. Save the configurations.

6. Exit and reboot PC to put the change into effect.

NOTE: If you had previously installed the driver in WFW and want to install

it from the diskette again, you must first delete the related OEM?.INF

file from the \WINDOWS\SYSTEM subdirectory.