***********************************************

*  CNet PCI-Bus CN930 Driver Installation     *

*  > Windows NT 3.5/3.51                      *

***********************************************

The driver for Windows NT 3.5/3.51 is located in the \WINNT\35 subdirectory

of the CN930 diskette.

1. Start Windows NT and logon as Administrator. From the Control Panel

select the "Network" icon.

2. From the Network window, click on the "Add Adapter..." button and choose

the item "<Other> Requires disk from manufacturer".

3. Insert this driver diskette into the appropriate drive.

4. Follow the instructions appearing on the screen and select the

appropriate drive name when required to give the location for the

driver and the OEMSETUP.INF file. Example:

A:\WINNT\35

5. After pressing the "Continue" button, the message "CNet CN930

Series PCI Ethernet Adapter" should be displayed. Click the OK button.

6. The screen will appear "Input EthernetID" dialog box which is provide

by E930.SYS driver. This option is only required when you have more

than one adapter on this computer.   Select SKIP if only one adapter

is installed on this computer. The files will automatically be copied to

the hard disk.

7. Now, the driver has been installed. Select the OK button from

the Network window. The protocols and driver will bind together.

8. Reboot.