***********************************************

*  CNet PCI-Bus CN930 Driver Installation     *

*  > SCO UNIX 5.X                             *

***********************************************

The SCO UNIX file is stored in the \SCOUNIX\5.X subdirectory of the CN930

diskette.

1.  Installation Diskette in DOS format.

Put CN930 SCO UNIX Driver diskette to floppy A: and use doscp

command to copy SCO UNIX PCI Driver into UNIX's directories.

# cd /

# doscp A:/scounix/5.x/setup  /setup

# chmod +x /setup

# ./setup

2.     Use netconfig utility

Available options:

# netconfig

3.  To add adapters, select "Add New LAN adapter" from "Hardware"

and select "CNet CN930 Series PCI Ethernet Adapter Driver".

4.  Add Protocol to select SCO TCP/IP.

5.  Enter the internet Address of this interface         n.n.n.n

6.  Enter the netmask for this interface                 (default 255.255.255.0)

7.  Enter the boardcast address for this interface       default: n.n.n.n

8.  Do you want to relink to kernel now ?               y

9.  Do you want this kernel to boot by default (y/n)    y

10. Do you want the kernel environment rebuilt (y/n)    y

11. Reboot unix.

# shutdown -y -g0