***********************************************

*  CNet PCI-Bus CN930 Driver Installation     *

*  > NetWare DOS ODI Workstation              *

***********************************************

The NetWare DOS ODI workstation driver file is stored in the \DOS_ODI

subdirectory of the CN930 diskette.

1. Power on the workstation, and execute Novell's LSL program.

2. At the DOS prompt type the following command to execute the NetWare 4.x

workstation driver --

[drive]:\DOS_ODI\E930

where [drive] is the floppy disk drive that this diskete is in.

3. After executing the driver, run NetWare's IPXODI.COM & NETX.EXE

(or VLM.EXE) files.

4. Please refer to Novell's Installation Manual for more details.